IME 461 / 561 — Discrete Event Simulation

Technical Report

Single Server Queuing System — Simio Simulation & Verification

Bradley University

Department of Industrial & Manufacturing Engineering

Simulation Tool:  Simio

Run Lengths Studied:  1 Day  ·  31 Days  ·  90 Days

Date:  March 2026

# 1.  Introduction

This report documents the construction, execution, and verification of a discrete-event simulation (DES) model of a single-server queuing system, implemented in Simio. The primary objective is to validate simulation outputs against the closed-form analytical results of M/M/1 queuing theory across three progressively longer simulation horizons: one day, 31 days, and 90 days. The system modeled represents a small pizza take-out store operating continuously, 24 hours a day, seven days a week.

Queuing theory provides exact steady-state performance measures under the assumptions of Poisson arrivals, exponentially distributed service times, and a single FIFO server. Simulation, while more flexible, is subject to transient start-up bias when executed without a formal warm-up period. Comparing both methods across increasing run lengths illustrates the convergence of simulation estimates toward theoretical values — a core concept in DES model verification.

# 2.  System Description

## 2.1  Queuing System Configuration

The system is classified as an M/M/1 queue under Kendall notation, with the following characteristics:

- Queue type: Single queue, single server (M/M/1)

- Queue discipline: First-In, First-Out (FIFO)

- Population assumption: Infinite source population

- Queue capacity: Unlimited (infinite)

- Operating schedule: Continuous — 24 hours/day, 7 days/week

- Physical context: Small pizza take-out store (analogous to street vendors or any single-attendant service point)

## 2.2  Arrival Process

Customer arrivals follow a Poisson process with a mean arrival rate of λ = 2 customers per minute, equivalent to exponentially distributed inter-arrival times with a mean of 0.5 minutes per customer.

Arrival rate:      λ  =  2 customers / min

Inter-arrival:     Exponential(mean = 1/λ = 0.5 min)

## 2.3  Service Process

The single server processes customers at a mean service rate of μ = 2.5 customers per minute. Service times are exponentially distributed with a mean of 0.4 minutes per customer.

Service rate:      μ  =  2.5 customers / min

Service time:      Exponential(mean = 1/μ = 0.40 min)

Number of servers: s  =  1

# 3.  Mathematical Model — M/M/1 Queuing Theory

The M/M/1 queuing model yields exact steady-state performance metrics when the traffic intensity ρ = λ/μ < 1. For the parameters above, ρ = 2.0 / 2.5 = 0.80, confirming a stable, non-saturating queue. The following closed-form expressions define the theoretical benchmarks used in this verification study:

Traffic intensity:            ρ   =  λ / μ         =  2.0 / 2.5    =  0.80  (80%)

P(system empty):              P₀  =  1 − ρ         =  1 − 0.80     =  0.20

Avg queue length:             Lq  =  ρ² / (1 − ρ)  =  0.64 / 0.20  =  3.200 customers

Avg no. in system:            L   =  Lq + ρ        =  3.200 + 0.800 =  4.000 customers

Avg waiting time in queue:    Wq  =  Lq / λ        =  3.200 / 2.0   =  1.600 min

Avg time in system:           W   =  Wq + 1/μ      =  1.600 + 0.400 =  2.000 min

P(customer waits):            ρ                                      =  0.80  (80%)

These values represent the long-run time-averaged performance that the simulation should approach as the run length increases and the warm-up transient diminishes.

# 4.  Simio Simulation Model

## 4.1  Model Architecture

The simulation model was constructed in Simio using the standard object library. The model consists of three primary objects connected in a linear flow path:

- Source (Entrance) — generates Customer entities with inter-arrival times sampled from Exponential(0.5 min), representing a Poisson arrival process at λ = 2 per minute.

- Server (PizzaTakeOutStore) — represents the single service counter. Service times are sampled from Exponential(0.4 min), corresponding to μ = 2.5 customers per minute. The server holds one resource unit and an unlimited input buffer queue operating under FIFO discipline.

- Sink (Exit) — destroys Customer entities upon service completion and accumulates flow-time and throughput statistics.

## 4.2  Simulation Run Parameters

Three separate simulation runs were conducted, all beginning with an empty-and-idle system state (no warm-up period applied). Simio records time in hours; all time-based outputs are converted to minutes (multiplied by 60) for comparison with M/M/1 theory expressed in minutes.

Table 0.  Simulation run configurations

| Run | Duration | Customers Processed |
| --- | --- | --- |
| Run 1 | 1 day (24 h) | 2,861 |
| Run 2 | 31 days (744 h) | 89,643 |
| Run 3 | 90 days (2,160 h) | 259,709 |

# 5.  Results and Verification

## 5.1  Performance Metrics — All Runs versus M/M/1 Theory

Table 1 presents all key performance metrics from the three simulation runs alongside the M/M/1 theoretical values. Utilization and probability figures are expressed as percentages; time metrics are expressed in minutes; queue and system content metrics are expressed in average number of customers. Maximum values are observable in simulation but have no steady-state theoretical counterpart.

Table 1.  Simulation versus M/M/1 theory — all run lengths

| Performance Metric | M/M/1 Theory | 1 Day | 31 Days | 90 Days |
| --- | --- | --- | --- | --- |
| Utilisation U (%) | 80.00% | 79.87% | 80.11% | 79.99% |
| L — avg no. in system | 4.000 | 3.596 | 4.038 | 3.940 |
| Lq — avg queue length | 3.200 | 2.797 | 3.237 | 3.141 |
| W — avg time in system (min) | 2.000 | 1.810 | 2.011 | 1.966 |
| Wq — avg time in queue (min) | 1.600 | 1.407 | 1.612 | 1.567 |
| P(0) — system idle probability | 0.200 | 0.201 | 0.199 | 0.200 |
| Max no. in system | — | 23 | 34 | 36 |
| Max time in system (min) | — | 9.90 | 15.00 | 14.99 |
| Customers processed | — | 2,861 | 89,643 | 259,709 |

Note: Simulation time outputs (originally in hours) multiplied by 60 to convert to minutes. Percentage errors for maximum values and observation counts are not applicable (shown as '—').

## 5.2  Convergence Analysis

Table 2 isolates the percentage deviation of each simulated metric from its M/M/1 theoretical value across all three run lengths. Red indicates underestimation, green indicates overestimation within tight bounds, and orange indicates moderate deviation.

Table 2.  Percentage deviation of simulation estimates from M/M/1 theory

| Metric | 1 Day | 31 Days | 90 Days |
| --- | --- | --- | --- |
| Utilisation U | -0.2% | +0.1% | <0.1% |
| L (avg in system) | -10.1% | +0.9% | -1.5% |
| Lq (queue length) | -12.6% | +1.2% | -1.8% |
| W (time in system) | -9.5% | +0.5% | -1.7% |
| Wq (time in queue) | -12.1% | +0.7% | -2.1% |

## 5.3  Discussion

Server Utilization

Server utilization is the most accurate and consistent metric across all three run lengths, deviating from the theoretical 80.00% by no more than 0.2% in any run. This accuracy is expected because utilization is a direct function of the λ/μ ratio — set by the input distributions — and is not substantially affected by the queue's transient build-up dynamics. As such, it converges rapidly regardless of run length.

Queue and System Content (L, Lq)

In the 1-day run, both L and Lq are substantially below theory (−10.1% and −12.6% respectively). This systematic underestimation results from warm-up transient bias: the simulation begins in an empty-and-idle state, and the queue has not yet reached its steady-state distribution by the end of the 24-hour run. With only 2,861 observations, the initial transient period constitutes a disproportionately large fraction of the total simulation time, pulling averages downward.

By 31 days (89,643 observations), errors fall to +0.9% and +1.2% respectively — well within acceptable verification bounds. The 90-day run produces errors of −1.5% and −1.8%, slightly larger than those of the 31-day run. This is attributable to statistical variability inherent in a single-replication study with no warm-up deletion; random fluctuation can shift results either direction around theory without indicating model error.

Waiting Times (W, Wq)

Waiting time metrics follow the same convergence pattern observed for content metrics. The 1-day run underestimates W and Wq by approximately 9–12%, while the 31-day run recovers to within 0.5–0.7% of theory. An important internal consistency check is provided by Little's Law (L = λW):

31-day run:  L = λW = 2.0 × 2.011 = 4.022  ≈  4.038  ✓

90-day run:  L = λW = 2.0 × 1.966 = 3.932  ≈  3.940  ✓

The near-exact agreement confirms internal consistency of the simulation model across all run lengths, independent of the absolute deviation from analytical theory.

Maximum Values

Maximum queue length and maximum time in system both increase monotonically with run length (maximum L: 23 → 34 → 36 customers). This behavior is expected — longer simulation runs provide more opportunity for rare, high-congestion events to occur. These maxima have no M/M/1 steady-state counterpart and serve as supplementary operational insights rather than verification criteria.

# 6.  Verification Summary

The Simio simulation model is verified as a valid representation of the M/M/1/∞/∞ queuing system. Key findings are as follows:

- The 1-day run exhibits systematic underestimation of all queue-related metrics by 9–13% due to warm-up transient bias from an empty-start condition. Server utilization remains accurate throughout.

- Extending the run to 31 days reduces all metric errors to within 1.2% of theoretical values, demonstrating strong convergence without applying a warm-up deletion period.

- The 90-day run achieves errors within 2.1% of theory; the marginal increase relative to the 31-day run reflects single-replication sampling noise rather than model deficiency.

- Little's Law holds internally for all three simulation runs, confirming model consistency.

- Simio correctly implements Poisson arrivals, exponential service times, FIFO queue discipline, and single-server operation as specified by the M/M/1 model assumptions.

To further improve estimate precision, future work should incorporate: (1) a warm-up period determined by Welch's method to eliminate initial-bias observations; (2) multiple independent replications to construct 95% confidence intervals; and (3) sensitivity analysis varying arrival rate toward saturation (ρ approaching 1.0) and extending to M/M/s configurations.

# 7.  Conclusion

This study verified a Simio discrete-event simulation model of a single-server M/M/1 queuing system by systematically comparing simulation outputs to analytical queuing theory across run lengths of 1 day, 31 days, and 90 days. Results confirm that simulation accurately reproduces theoretical steady-state behavior when the run length is sufficiently long relative to the system transient period. The progressive convergence from errors exceeding 12% in the 1-day run to errors below 1.2% in the 31-day run provides clear empirical evidence of the critical role of run-length selection in discrete-event simulation practice. The verified model constitutes a reliable baseline for subsequent experimentation, including multi-server configurations (M/M/s), arrival-rate sensitivity sweeps, and warm-up period optimization.

# References

Banks, J., Carson, J. S., Nelson, B. L., & Nicol, D. M. (2010). Discrete-Event System Simulation (5th ed.). Pearson Prentice Hall.

Gross, D., Shortle, J. F., Thompson, J. M., & Harris, C. M. (2008). Fundamentals of Queueing Theory (4th ed.). John Wiley & Sons.

Law, A. M. (2015). Simulation Modeling and Analysis (5th ed.). McGraw-Hill Education.

Simio LLC. (2024). Simio Reference Guide. Retrieved from https://www.simio.com